Virtual Network Simulation: Design, Monitoring and Troubleshooting

Building, monitoring and troubleshooting a routed network in Cisco Packet Tracer

| Prepared by | Elyas Ghwiet |
| --- | --- |
| Program | Yuva virtual internship - IT / Network Support task |
| Simulation software | Cisco Packet Tracer |
| Date | October 2026 (first week) |

Executive summary

This report documents a small-scale virtual network built in Cisco Packet Tracer to practise establishing, maintaining and troubleshooting network systems. The network joins two LANs through two routers running RIPv2. Monitoring was implemented with ping, traceroute, Syslog and SNMP, and packet flow was inspected in Simulation mode. Three realistic faults (a shut-down router interface, a wrong default gateway and a removed routing process) were introduced, diagnosed and resolved, and the evidence for each is shown in the sections below.

Topology: 2 routers, 2 switches, 4 PCs and 1 server across two subnets joined by a /30 link.

Monitoring: ICMP (ping), traceroute, Syslog to Server0, SNMPv2c queries from PC1, and Simulation-mode packet capture.

Troubleshooting: 3 incidents documented with symptom, diagnosis, root cause, resolution and verification.

1. Introduction and objectives

Network maintenance depends on three habits: knowing what a healthy network looks like, watching it continuously, and following a repeatable method when something breaks. The objective of this task was to build a small environment where all three can be practised safely.

Select suitable simulation software and justify the choice.

Design a topology with routers, switches and at least two end nodes.

Implement monitoring tools (ping, traceroute, SNMP, Syslog) inside the simulation.

Simulate network issues, then document the troubleshooting steps and resolutions.

Compile the configuration, evidence and analysis into this report.

2. Simulation software selection

Three common options were compared before starting.

| Criterion | Cisco Packet Tracer | GNS3 | Virtual machines (VirtualBox / VMware) |
| --- | --- | --- | --- |
| Cost | Free with a Networking Academy account | Free, but needs router images | Free hypervisor, OS licences vary |
| Hardware needs | Low (runs on a normal laptop) | Medium to high | High (one VM per device) |
| Learning curve | Gentle; graphical, built-in devices | Steeper; image and server setup | Steep; manual OS and network setup |
| Router / switch CLI | Cisco IOS-style CLI | Real IOS images | Depends on software router |
| SNMP, Syslog, Simulation mode | Built in (MIB Browser, Syslog service, packet capture) | External tools needed | External tools needed |

Decision: Cisco Packet Tracer was selected. It provides everything the task requires in a single tool - IOS-style configuration, an SNMP MIB Browser, a Syslog service on the server and a packet-level Simulation mode - and it runs comfortably on a standard laptop. Its main limitation is that devices are simulated, so a few advanced IOS features are unavailable; none of them were needed here.

3. Network design

3.1 Topology

The design uses two LANs, each with its own switch and default gateway, connected by a point-to-point /30 link between two routers. A server in LAN1 acts as the Syslog collector, and PC1 acts as the SNMP management station. Keeping the monitoring tools in LAN1 means traffic from Router2 must cross the router link to reach them, which also tests routing.

Figure 1: Network topology as built in Cisco Packet Tracer (Logical view).

Figure 2: Logical topology with interfaces and IP addressing.

3.2 Device inventory

| Device | Model | Hostname | Role |
| --- | --- | --- | --- |
| Router1 | Cisco 2911 | R1 | Default gateway for LAN1; RIPv2; SNMP agent; Syslog client |
| Router2 | Cisco 2911 | R2 | Default gateway for LAN2; RIPv2; SNMP agent; Syslog client |
| Switch1 | Cisco 2960-24TT | - | Layer 2 access switch for LAN1 |
| Switch2 | Cisco 2960-24TT | - | Layer 2 access switch for LAN2 |
| PC1, PC2 | PC-PT | - | LAN1 clients; PC1 is also the SNMP manager |
| PC3, PC4 | PC-PT | - | LAN2 clients |
| Server0 | Server-PT | - | Syslog collector for both routers |

3.3 IP addressing plan

| Device | Interface | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 (to Switch1) | 192.168.1.1 | 255.255.255.0 | - |
| R1 | Gi0/1 (to R2) | 10.0.0.1 | 255.255.255.252 | - |
| R2 | Gi0/0 (to Switch2) | 192.168.2.1 | 255.255.255.0 | - |
| R2 | Gi0/1 (to R1) | 10.0.0.2 | 255.255.255.252 | - |
| PC1 | FastEthernet0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | FastEthernet0 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| Server0 | FastEthernet0 | 192.168.1.100 | 255.255.255.0 | 192.168.1.1 |
| PC3 | FastEthernet0 | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC4 | FastEthernet0 | 192.168.2.11 | 255.255.255.0 | 192.168.2.1 |

3.4 Cabling plan

| From | To | Cable type |
| --- | --- | --- |
| PC1, PC2, Server0 | Switch1 (Fa0/2, Fa0/3, Fa0/4) | Copper straight-through |
| Switch1 Fa0/1 | Router1 Gi0/0 | Copper straight-through |
| Router1 Gi0/1 | Router2 Gi0/1 | Copper cross-over (router to router) |
| Router2 Gi0/0 | Switch2 Fa0/1 | Copper straight-through |
| PC3, PC4 | Switch2 (Fa0/2, Fa0/3) | Copper straight-through |

Design choices. RIP version 2 was chosen because it exchanges routes automatically, so a failed link or a removed routing process shows up immediately in the routing tables - ideal for the troubleshooting exercises. A /30 subnet is used for the router link because it needs only two usable addresses. Switches run with default settings since both LANs are single-VLAN.

4. Configuration

All devices were configured from the CLI (routers) or the IP Configuration panel (hosts). The complete command listings are in Appendix A; the essential parts are summarised here.

4.1 Interfaces and routing

Router1 (Router2 is configured symmetrically with 192.168.2.1 and 10.0.0.2)

R1(config)# interface gigabitEthernet 0/0 R1(config-if)# ip address 192.168.1.1 255.255.255.0 R1(config-if)# no shutdown R1(config)# interface gigabitEthernet 0/1 R1(config-if)# ip address 10.0.0.1 255.255.255.252 R1(config-if)# no shutdown R1(config)# router rip R1(config-router)# version 2 R1(config-router)# network 192.168.1.0 R1(config-router)# network 10.0.0.0 R1(config-router)# no auto-summary

4.2 Monitoring services

Applied on both routers

! Syslog: send router logs to Server0 logging host 192.168.1.100 ! SNMP: read-only and read-write community strings snmp-server community public ro snmp-server community private rw

On Server0 the SYSLOG service was switched on (Services tab). PC1 uses the built-in MIB Browser with address 192.168.1.1 or 192.168.2.1, read community public and SNMP version v2c.

5. Monitoring and verification

5.1 Interface and routing tables

The first check after configuration is whether interfaces are up and whether routes are learned. On Router2 both configured interfaces report up / up; the third interface and Vlan1 are unused and administratively down by default.

Figure 3: Router2: show ip int brief - Gi0/0 (192.168.2.1) and Gi0/1 (10.0.0.2) are up/up.

On Router1 the routing table contains the two connected networks (C and L entries) and a R entry, proving that RIP learned the LAN2 prefix from Router2 across the 10.0.0.0/30 link. The administrative distance and metric are [120/1], i.e. RIP, one hop away.

Figure 4: Router1: show ip route - 192.168.2.0/24 learned via RIP from 10.0.0.2.

5.2 Ping and traceroute

Ping verifies reachability and round-trip time; traceroute shows the path hop by hop. From PC1 to PC3:

Ping: replies arrive from 192.168.2.10 with TTL=126 (initial value 128 minus two router hops). The first request timed out because PC1 had to resolve the gateway MAC address with ARP; the remaining requests succeeded (3/4 received).

Traceroute: three hops - 192.168.1.1 (Router1), 10.0.0.2 (Router2) and the destination 192.168.2.10 - confirming the path.

Figure 5: PC1: ping and tracert to PC3 (192.168.2.10).

5.3 Syslog

Both routers send their event messages to Server0. To generate real events, the Gi0/1 interface on Router1 was shut down and brought back up. The Syslog table below shows the configuration change and the link and line-protocol transitions as they were reported, together with the source address of each router.

Figure 6: Server0 Syslog service receiving messages from Router1 (192.168.1.1) and Router2 (10.0.0.2).

| Message | Meaning |
| --- | --- |
| %SYS-6-LOGGINGHOST_STARTSTOP ... started | Router began sending logs to 192.168.1.100 on UDP port 514 |
| %SYS-5-CONFIG_I: Configured from console | Someone changed the configuration from the console |
| %LINK-5-CHANGED ... administratively down | Interface Gi0/1 was shut down manually |
| %LINEPROTO-5-UPDOWN ... down | Layer 2 protocol on the interface stopped |
| %LINK-5-CHANGED ... up / %LINEPROTO ... up | Interface and protocol recovered after no shutdown |

Severity levels are encoded in the message code: level 5 is a notification and level 6 is informational.

5.4 SNMP

SNMPv2c was enabled on both routers. From PC1, the MIB Browser sent Get requests to each router. The queried objects belong to the standard MIB-2 system group:

| Object | OID | Result |
| --- | --- | --- |
| sysDescr.0 | .1.3.6.1.2.1.1.1.0 | Cisco IOS Software, C2900 Software (C2900-UNIVERSALK9-M) |
| sysName.0 | .1.3.6.1.2.1.1.5.0 | R2 (queried on 192.168.2.1) |

Figure 7: SNMP Get of sysDescr.0 from Router1 (192.168.1.1).

Figure 8: SNMP Get of sysName.0 from Router2 (192.168.2.1).

During setup, the first SNMP request returned Request timed out even though ping to the router worked, which means the network path was fine and the problem was in the SNMP settings. After checking the agent configuration on the router and the community string and version in the manager's Advanced settings, the requests succeeded. Typical causes of this symptom are a missing snmp-server community line on the router or a community string or version mismatch on the manager.

5.5 Packet-level view (Simulation mode)

Simulation mode, filtered to ICMP, was used to watch a ping from PC1 to PC3 move through the network. The event list records each device the packet visits and the time elapsed. The packet crosses Switch1, Router1, Router2 and Switch2 and the echo reply returns along the same path; the PDU list reports the final status Successful. As on the real network, the first attempt may fail while ARP entries are learned.

Figure 9: Simulation mode: event list for ICMP between PC1 and PC3, final status Successful.

5.6 Monitoring summary

| Tool | What it shows | Where used | Outcome |
| --- | --- | --- | --- |
| ping | Reachability, latency, TTL | PC1, PC3 | End-to-end connectivity confirmed |
| tracert | Hop-by-hop path | PC1 | Path via 192.168.1.1 and 10.0.0.2 |
| show ip interface brief | Interface status | R1, R2 | All used interfaces up/up |
| show ip route | Learned and connected routes | R1, R2 | RIP routes present |
| Syslog | Device events in one place | Server0 | Link and config events collected |
| SNMP (MIB Browser) | Device identity and status | PC1 | Both routers queried remotely |
| Simulation mode | Packet path and timing | Whole network | Packet flow visualised |

6. Troubleshooting

6.1 Method

Each fault was handled with the same repeatable process:

Baseline: record that the network works before changing anything.

Identify: describe exactly what fails and what still works.

Isolate: use the evidence to place the fault on a device or layer (host, switch, router, link).

Fix: apply the smallest change that removes the root cause.

Verify: repeat the original failing test and confirm the network is healthy again.

6.2 Incident 1 - Router interface shut down

Figure 10: Baseline before the fault: PC1 pings PC3 with 0% loss.

Fault injected on Router1

R1# configure terminal R1(config)# interface gigabitEthernet 0/1 R1(config-if)# shutdown

Figure 11: Symptom: ping from PC1 to PC3 fails with 100% loss.

| Item | Detail |
| --- | --- |
| Symptom | Ping from PC1 to PC3 failed (4 sent, 0 received). The replies were Destination host unreachable and came from 192.168.1.1. |
| Analysis | A reply from 192.168.1.1 means the packet reached Router1 - so PC1, the cable and Switch1 are fine - and Router1 itself reported that it has no route to 192.168.2.0/24. The fault is therefore at Router1 or on the link to Router2, not in LAN1. |
| Root cause | Interface GigabitEthernet0/1 on Router1 (the link to Router2) was administratively shut down. With the interface down, Router1 lost the RIP route to 192.168.2.0/24. |
| Resolution | Re-enable the interface with no shutdown; RIP then re-learned the route. |
| Verification | The interface returned to up/up, show ip route shows the route to 192.168.2.0/24 again (learned 15 seconds earlier) and ping from PC1 to PC3 succeeds. |

Fix

R1# configure terminal R1(config)# interface gigabitEthernet 0/1 R1(config-if)# no shutdown

Figure 12: After the fix: interface up and the RIP route to 192.168.2.0/24 restored on Router1.

Figure 13: Verification: ping from PC1 to PC3 succeeds again (4/4, TTL=126).

6.3 Incident 2 - Wrong default gateway on a host

The default gateway on PC3 was changed from 192.168.2.1 to 192.168.2.254, an address that does not exist.

Figure 14: Symptom: PC3 reaches PC4 in its own LAN (TTL=128) but pings to PC1 time out.

| Item | Detail |
| --- | --- |
| Symptom | PC3 could ping PC4 in the same subnet but could not reach anything outside LAN2 (request timed out). |
| Analysis | Local traffic works, so the NIC, cable and Switch2 are fine. Remote traffic fails, which points to the default gateway. Unlike Incident 1, no device answered at all: the packets were addressed to a gateway that does not exist. |
| Diagnosis | Ping to the configured gateway 192.168.2.254 timed out, while ping to the real router interface 192.168.2.1 succeeded (TTL=255, i.e. a router). The router is healthy; only the gateway setting on PC3 is wrong. |
| Root cause | Incorrect default gateway (192.168.2.254) configured on PC3. |
| Resolution | Set the default gateway to 192.168.2.1 in IP Configuration. |
| Verification | ipconfig shows 192.168.2.1 as gateway and ping from PC3 to PC1 succeeds (TTL=126). |

Figure 15: Diagnosis: the wrong gateway 192.168.2.254 does not answer.

Figure 16: Diagnosis: the real router interface 192.168.2.1 answers (TTL=255).

Figure 17: Verification: corrected gateway in ipconfig and successful ping to PC1.

6.4 Incident 3 - Routing process removed from a router

Fault injected on Router2

R2# configure terminal R2(config)# no router rip

| Item | Detail |
| --- | --- |
| Fault | The RIP process was removed from Router2. Router2 stops exchanging routes and discards the routes learned from Router1, so it can no longer forward traffic to 192.168.1.0/24. |
| Expected indicators | show ip route on Router2 shows only connected (C) and local (L) entries and no R entry for 192.168.1.0/24; show ip protocols no longer lists RIP. |
| Root cause | Missing routing configuration on Router2 (dynamic routing disabled), not a physical or addressing problem - all interfaces stayed up. |
| Resolution | Re-create the RIP configuration on Router2 (commands below) and save it. |
| Verification | Router2 again lists 192.168.1.0/24 as a RIP route via 10.0.0.1, learned a few seconds after the configuration was re-applied (see figure), so forwarding between the two LANs is possible again. |

Fix

R2# configure terminal R2(config)# router rip R2(config-router)# version 2 R2(config-router)# network 192.168.2.0 R2(config-router)# network 10.0.0.0 R2(config-router)# no auto-summary R2(config-router)# end R2# write memory

Figure 18: After the fix: Router2 learns 192.168.1.0/24 via RIP from 10.0.0.1.

6.5 Incident summary

| # | Fault | Layer / area | Key evidence | Fix |
| --- | --- | --- | --- | --- |
| 1 | Router interface shut down | Network / interface | Unreachable reply from 192.168.1.1; route missing | no shutdown |
| 2 | Wrong default gateway on PC3 | Network / host configuration | Local ping OK, remote ping times out | Gateway set to 192.168.2.1 |
| 3 | RIP removed from Router2 | Network / routing | No R entries in routing table | Re-create router rip |

7. Conclusion and recommendations

The simulation met every objective: a working two-LAN routed network was designed and configured, monitored with ping, traceroute, Syslog, SNMP and Simulation mode, and three different classes of fault were diagnosed and fixed using evidence rather than guesswork. The most useful lessons were:

The source of an error message matters. The unreachable reply from the gateway in Incident 1 immediately narrowed the fault to Router1, while the silence in Incident 2 pointed at the gateway setting.

Comparing what still works (same-subnet ping, router ping) with what fails is the quickest way to locate a fault.

Centralised logging and SNMP give visibility without logging in to every device.

A failed first ping is often just ARP; re-test before concluding that something is broken.

Recommended improvements for a production network:

Replace the default public/private SNMP community strings, remove the read-write community and prefer SNMPv3.

Restrict SNMP and Syslog access with access lists and add NTP so log timestamps are meaningful.

Add a redundant link between the routers so that one failed interface does not split the network.

Save configurations after every change and keep a backup copy (Appendix A).

Appendix A - Configuration commands

The listings below contain the commands applied to each device. They are the working configuration scripts for this lab and can be pasted into the CLI of an equivalent topology.

Router1 (R1)

enable configure terminal hostname R1 interface gigabitEthernet 0/0 ip address 192.168.1.1 255.255.255.0 no shutdown exit interface gigabitEthernet 0/1 ip address 10.0.0.1 255.255.255.252 no shutdown exit router rip version 2 network 192.168.1.0 network 10.0.0.0 no auto-summary exit logging host 192.168.1.100 snmp-server community public ro snmp-server community private rw end write memory

Router2 (R2)

enable configure terminal hostname R2 interface gigabitEthernet 0/0 ip address 192.168.2.1 255.255.255.0 no shutdown exit interface gigabitEthernet 0/1 ip address 10.0.0.2 255.255.255.252 no shutdown exit router rip version 2 network 192.168.2.0 network 10.0.0.0 no auto-summary exit logging host 192.168.1.100 snmp-server community public ro snmp-server community private rw end write memory

End devices (static addressing)

| Device | IP address | Mask | Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| Server0 (Syslog ON) | 192.168.1.100 | 255.255.255.0 | 192.168.1.1 |
| PC3 | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC4 | 192.168.2.11 | 255.255.255.0 | 192.168.2.1 |

Appendix B - Command reference

| Command | Purpose |
| --- | --- |
| show ip interface brief | Interface status and IP addresses |
| show ip route | Routing table (C, L, R entries) |
| show ip protocols | Routing protocols running on the router |
| show logging | Local log buffer and Syslog settings |
| ping <ip> | Test reachability and round-trip time |
| tracert <ip> | Show the path to a destination (PC command prompt) |
| ipconfig | Show host IP address, mask and gateway |
| interface <x> / shutdown / no shutdown | Disable or enable an interface |
| write memory | Save the running configuration |